- Purpose: The purpose of the Sanitation SOP’s is to provide Miller sanitation personnel the procedures to safely and effectively clean the facility and equipment.

- Scope: Double-Under Conveyor belt & Short Conveyor Belt.

- Definitions

  - None

- Responsibilities

  - Sanitarian: Clean, rinse, and sanitize the equipment

  - Pre-op Personnel: Visually inspect, swab, reclean if necessary

  - VP of Administrations: Assure adherence to sanitation SOP’s and related documentation.  Review and update as necessary.

- Procedure

  - Put on the appropriate PPE for the sanitation chemicals in use

  - Turn off power at disconnect on wall and lockout/tagout. (Safety purpose)

  - Scrape major debris from hopper cone, side tables, control switch, motor, sprockets, sliding conveyor & conveyor belt. (Detached side tables to clean underneath for any harborage)

  - Rinse with hot, low-pressure water hopper cone, side tables, sprockets, sliding conveyor & conveyor belt and lift belts from the top to the bottom to remove excess food residues.

  - Foam the hopper cone, side tables, control switch, motor, sprockets, sliding conveyor & conveyor belt. with Enrich 299 at the recommended concentration and let it sit for 15-20 minutes.

  - Rinse with hot, low-pressure water to remove chemical residues

from the top to bottom.

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize the entire Double-Under Conveyor belt & Short Conveyor belt with Multi-Quat at the recommended concentration.

  - Return power to the Double/Under Conveyor belt and remove lockout/tagout.

- For weekly: Use Perasan A. to sanitize entire Double-Under Conveyor belt & Short Conveyor Belt.

Mixing Procedure & measurements.

    - To mix please use the following based on 25 gallons of water:

    - Perafoam- Mix 6 ounces every 6 gallons of water. (20 ounces total)

    - Perasan A- Mix 1 ounces for every 6 gallons of water (4 ounces total)

    - Titrate using the Perasan A testing procedure, target is 91-195ppm of PerasanA.(+/- 13ppm)

- Records

  - Pre-op checklist to include Charm unit swab record.

  - Master Sanitation Plan

- Document History

| REVIEWED: | PROPOSED CHANGE: |
| --- | --- |
| Roland Pilar 9/28/2022 | Added Perasan A mixing procedure & measurement. |
| Roland Pilar / Robles 03/15/2023 | Added SSOP number |