	STANDARD MANUFACTURING PROCEDURE
Flushing Ingredient Bins and System for Bangladesh
	Effective Date
4/20/2021
Reviewed

1/9/2022
	Number
WES-E-503

	Agpack LLC. - Westville, OK 
	Page

1 of 4
	Revision

#0



CONCEPT  

The following information is for how to perform a flush of the ingredient bins and the mixing system specifically for production of product being exported to Bangladesh.
I. SAFETY CONCERNS 

Flying debris (dust) 

       Safety Equipment Needed

· Safety glasses
· Dust mask

 Hazards

A. While loading the flush material it is possible that wind could create a flying debris hazard.
II. PRIOR KNOWLEDGE:

All Agpack LLC. employees involved in an ingredient bin and system flush should be trained on manual operation of the batching & loadout systems.
IV   
    PROCEDURES 
A. When flushing the receiving system: 2000lbs of feather meal will be flushed through the system and into a segregated bin that will be treated as a prohibited product. 2000lbs represents one full exchange of the receiving pit/augers capacity.
B. To flush the mixing system:

1. 6000lbs of feather meal from bin #3 will be manually augured into the weigh hopper. Weigh hopper has a 6000lb capacity.

2. The feather meal will be cycled into the mixer and allowed to mix.

3. The loadout system will be placed on rail loadout.

4. An empty railcar will be placed under the loadout system.

5. The flush product will then be dumped into the surge and flushed through the loading system and out to the car.
6. The feather meal will then be unloaded into an isolated feather bin #15 and treated as prohibited for export to Bangladesh. This product from bin #15 will be consumed in appropriate formulation to approved customers.

7. The railcar will then be cleaned.

C. In both flush procedures the amount of flush material must equal to the full capacity of the system being flushed to achieve one complete change of operating volume.
D. Each System Flush for Bangladesh shall be recorded, and documentation shall be maintained.
V.
Quality concerns: 

Due to the quality that Agpack LLC. guarantees to its customers it is very important that these steps be followed while doing a flush.
Training Documentation

Employee Name

Date of Training/Certification

Signature verifying trainee      










has read and understands       










SOP

	
	
	
	Bangladesh Flush
	

	DATE
	TIME
	PRODUCT
	VOLUME
	DESTINATION
	OPERATOR

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	


