	[image: image1.png]L =

il R (Gratirs ~ Do)

Deparimert Approval
V s )

Al Rviow (Shnatirs - Oe)

Aol oviow (Sgrafo~ Dot

Al Reviow (Sinatars - 0e)

Al Raviow (Snaes ~ Ose)

il Reviow (Signatrs - Do)





	
	Customer Complaints



	Quality Assurance System

Operating Procedures


	OP-00-154-DCS

	Prepared By:  Angie Zibilski
	

	Supersedes – March 30, 2000
	Effective – October 5, 2000
	Page 5 of 5

	


	[image: image2.png]«
CRIMSON

oLDINGS




	
	Cleaning and Sanitizing All Plant Floors

	
	
	CP 114 - ADR

	
	

	Supersede Date: 12/27/21
	Revised: 12/28/2023
	Page 1 of 5



Changes

1. Changed the chemical used from Enforce LP to Pathoclean.
[image: image3.emf]
I PURPOSE

A.  To clean and sanitize floors in processing, bulk liquid product storage areas, and receiving bays on a routine basis to control microbiology growth and thus ensure food safety.
II DEFINITIONS

III PLANT SPECIFIC RESPONSIBILITIES

A. Maintain a Master Sanitation Schedule and provide forms to document completed tasks.

B. Provide appropriate cleaning chemicals.
	Product
	Concentration
	Temperature
	Time

	Manual Wash
	
	
	

	Pathoclean
	2 – 5 ounce/gallon water
	Ambient
	5 – 15 minutes

	
	
	
	

	Sanitize
	
	
	

	Ster-Bac
	1 ounce/gallon water

400-800 ppm
	Ambient
	Allow to Dry

	
	
	
	


IV MATERIALS

A. Pathoclean central foaming station
B. Ster Bac
C. Floor Scrubber

D. Chemical PPE

E. Yellow Brushes

F. 2.5 Gallon Pump Sprayer

G. Ster-Bac

V PROCEDURE

A. Cleaning of floors
1. Clean floors per frequency defined by the Master Sanitation Schedule.  Monthly task sheets define tasks to be completed in the designated current month.  Task sheets are posted in various work areas. 
2. Remove and dispose of debris from floor surfaces.
3. Rinse the floor area with hot water.
4. Operator applies Pathoclean foam to the floors.
a. Operator to have in place all appropriate PPE such as safety glasses, face shield, green gloves, and apron.
5. Manually scrub floors thoroughly using a yellow bristle brush or powered scrubber.
6. Rinse the floor area with water.
7. Proceed to sanitizing step as defined below. 
B. Sanitizing of Floors
1. Thoroughly rinse the floors with cold water.
2. Prepare a solution of Ster-Bac Sanitizer by mixing 2 ounce of Ster-Bac concentrate with 2 gallons of cold water.  This blend ratio will create an 800-ppm quaternary solution. 
3. Add the above blended sanitizer solution to a 2.5-gallon pump sprayer and thoroughly spray the entire floor area with the Ster-Bac Sanitizer blended solution. 
4. Document completion of task on the Master Sanitation Schedule monthly task sheet for the designated current month.  Date and initial the monthly task sheet appropriately.
VI REPORTING

A.  Report any physical issues with floors to Production Supervisor and complete a work order for Maintenance.
VII REFERENCE

A. Adrian Master Sanitation Schedule 

B. SDS

VIII RECORD RETENTION

A. The Operations Manager and Production Supervisors distributes and retrieves the monthly task sheets for the designated current month to complete a monthly accountability report for the Master Sanitation Schedule program.

B. Records will be kept according to the current record retention policy.
This procedure has been read and understood by the following personnel:

	Employee #
	Employee Name
	Date
	Employee #
	Employee Name
	Date

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	


Commercial, Confidential Trade Secret of Crimson Holdings LLC, Do not copy or distribute without prior written consent from Crimson Holdings LLC 


[image: image2.png]