- Purpose: The purpose of the Sanitation SOP’s is to provide Miller sanitation personnel the procedures to safely and effectively clean the facility and equipment.

- Scope: Vemag Stuffer-Linkers: 208 Servo, 209 and 218

- Definitions

  - Stuffing Screws: Twin screws within the body of the stuffer that gently meter product from the bottom of the hopper through the barrel.

- Responsibilities

  - Sanitarian: Clean, rinse, and sanitize the equipment

  - Maintenance: Hand clean control and vacuum pump compartment of stuffer quarterly.  Hand clean control compartment for linker quarterly.

  - Pre-op Personnel: Visually inspect, swab, reclean if necessary

  - VP of Administration: Assure adherence to sanitation SOP’s and related documentation.  Review and update as necessary.

- Procedure

  - Put on the appropriate PPE for the sanitation chemicals in use

  - Disconnect machine power. Disconnect air from machine.

  - Separate link conveyor from linker. Separate linker from stuffer.

  - Release stuffer hopper and tilt back (2 people)

  - Remove feed screw at bottom of hopper

  - Remove hopper gasket

  - Remove vacuum filter cover, filter assembly, and O-ring

  - Remove stuffing horn

  - Remove twin stuffing screws

  - Remove stuffing tubes

  - Remove linking chain

  - Soak items from 5.6 through 5.12 in Enrich 299 for one hour

    - Hand wash parts with single use scratch pad or brush.

    - Rinse parts with low pressure hot water.

    - Sanitize parts with Multi-Quat.

  - Rinse stuffer, linker, links conveyor with low pressure, hot water from top to bottom.

  - Foam stuffer, linker, links conveyor with Enrich 299 at recommended concentration.

  - Handwash stuffer, linker, links conveyor with single use scratch pad of while bristled brush.

  - Rinse stuffer, linker, links conveyor with low pressure, hot water from top to bottom.

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize stuffer, linker, links conveyor with Multi-Quat at the recommended concentration.

  - Reinstall stuffer and linker parts, reattach linker to stuffer, and links conveyor to linker.

- Records

  - Pre-op checklist to include Charm unit swab record.

  - Sanitation Master Plan

- Document History

| REVIEWED: | PROPOSED CHANGE: |
| --- | --- |
| Roland Pilar 12/7/2021. | Removed Iodosan 485 changed Multi-Quat Added Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed) |
| Roland Pilar / Alfredo Robles 03/15/2023 | Added SSOP number |