- CONCEPT

The following information is to describe and outline the pest control program.

### PRIOR KNOWLEDGE

The employee must read and understand the procedures on this SOP.           .

- SAFETY CONCERNS

### Do not improperly handle Pest Control Chemicals. Follow SDS instructions and good handwashing practices.

- QUALITY CONCERNS

This SOP applies to the efforts required to control and eliminate pests from entering the facility and contaminating product and raw material in all production and warehouse areas.

### PROCEDURES

The Plant Manager is responsible for overseeing and directing the pest control program as well as reviewing any proposed pesticide use. The pest control contractor is responsible for adhering to Agpack requirements and conducting annual verification.

References

21 CFR Part 110

Materials/Equipment

- Pest Control Devices

- Approved Chemical List

- Certificate of Insurance

- PCO license

- Pest Control Device Map

- SDS and Pesticide Labels

Monitoring and Inspection for Pests

Monitoring for pests (insects and rodents) and vermin (domestic and feral animals and birds) shall be conducted through use of a contract pest service and through employee vigilance.

The pest control service shall:

- Licensed and approved by the facility.

- Use only trained and qualified operators that comply with regulatory requirements.

- Provide their current name, address, and license number.

- Perform monthly inspections for pest activities.

- Change ILT bulbs on an annual basis, if applicable.

- Provide a pest control management plan to include a facility map indicating numbered locations of the bait stations, traps and ILTs.

- Provide a written record of any observations of pest activity, gaps or access ways for pests to enter the facility, or other pest control concerns and control measures used.

- Report to a responsible authorized person on entering the premises and after the completion of inspections or treatments.

Agpack employees shall:

- Plant Manager shall issue maintenance work orders for any observations noted by the pest service.

- Employees who observe pests shall report the pest activity to the office.

- The situation shall be evaluated by the Plant Manager to determine whether or not the pest control service shall be called and if production shall continue in the area.

- Employees shall be made aware of the use of pest and vermin control chemicals and the measures to take when they come into contact with a bait station.

Control Devices

Control devices may be used as a preventative or reactive tool for eliminating pests from the facility.

Control devices may include:

- Mechanically operated traps and bait stations for rodents.

- Electronic or glue board traps for insects.

Traps or other control devices shall:

- Be emptied routinely and cleaned as necessary.

- Installed and operated away from processing areas.

- Maintained in good repair.

Control Measures

General maintenance and operating practices shall be utilized to discourage the entry of pests into the facility.

Such practices include:

- Keeping exterior doors and windows closed when not in use.

- Repairing holes and other routes where pests may gain entrance.

- Inspecting incoming shipments for indications of pests.

- Keeping food waste away from buildings and emptying waste containers often.

Use of Pesticides

The use of pesticides and poison baits shall be closely controlled to assure that contamination with product and raw materials will not occur.

- Only licensed pesticide applicators may apply pesticides at the facility.

- No pesticides or poison bait shall be used by the authorized pest control contractor without authorization by the Plant Manager.

- All pesticides that are approved shall be documented by name and composition and have an SDS and labels on file.

- All pesticides used in areas where raw materials, packaging, product, or contact equipment is present must be approved for use.

- Pesticides shall not be stored on site and empty pesticide containers shall not be discarded on site.

- As applicable, all organic raw materials and finished products must be removed from the plant during a fogging or fumigation pesticide treatment.  No organic raw materials or finished goods may be in the plant for five days following the treatment.

- Appropriate cleaning, as prescribed by the manufacturer, shall be performed in the immediate area and on any nearby equipment if pesticides are used in GMP areas.

- Poison bait shall only be used outside of the building.

Effectiveness Checks

The pest contractor management shall provide an annual effectiveness check of the program to verify the elimination of applicable pests.

Records

The pest inspection reports, record of any pesticides or poison bait used, SDS sheets and product labels for approved pesticides/bait, contract service information, maps, and any other pest control information shall be maintained at the plant level.

##### TRAINING DOCUMENTATION

Employee Name	Date of Training/Certification		Signature verifying trainee      									has read and understands SOP

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

- REVISION HISTORY

| REVISION NUMBER | DATE | ORIGINATOR | REASON |
| --- | --- | --- | --- |