P242-006-01 Vacuum Filler Cleaning

Equipment Cleanout Instructions and Log

Equipment Name: Vacuum Filler for Extracts and Flavors

Frequency: When changing lot numbers

Procedure:

- Spray machine with water and wipe with new paper towels.

- Inspect glass overflow jugs for chips or cracks in neck area.  If found, follow procedure for broken glass and replace jug.

- Fill 5 gallon bucket to fill line with warm water and add ¼ cup bleach.

- Use test strip to check that chlorine level is 100 – 200 ppm

- Set up machine and run until glass overflow jug is full and discard mixture.

- Repeat step 5.

- Fill clean bucket with fresh warm water.

- Set up machine and run until glass overflow jug is full and discard mixture.

- Repeat step 8 and check water in overflow with residual chlorine test strip to make sure level is less than 5 ppm.  If not, repeat step 8 until it is.

- Empty all container and lines of water residue.

- Mark date of cleanout on F-242-006 Cleaning Log noting the old lot number in the “Lot Number #1” column and the new lot number in the “Lot Number #2” column.