- Purpose: The purpose of the Sanitation SOP’s is to provide Miller sanitation personnel the procedures to safely and effectively clean the facility and equipment.

- Scope: Seydelmann Bowl Chopper (“Old” and “New”)

- Definitions: None

- Responsibilities

  - Sanitarian: Clean, rinse, and sanitize the equipment

  - Maintenance: Hand cleans cabinet under the bowl quarterly.  Maintenance disassembles and reassembles the blade assembly weekly to allow more thorough cleaning in this area and sharpening of blades.

  - Pre-op Personnel: Reassemble blades and drain plugs, visually inspect, swab, reclean if necessary

  - VP of Administration: Assure adherence to sanitation SOP’s and related documentation.  Review and update as necessary.

- Procedure (Daily)

  - Put on the appropriate PPE for the sanitation chemicals in use

  - Turn off power and lockout.

  - Open lid

  - Raise discharge disc.

  - Put plastic bag over Seydelmann control panel

  - Scrape major debris from bowl, lid, blades, discharge disc, and discharge chute with stainless putty knife or similar.

  - Remove two drain plugs in bottom of bowl rinse handwipe drain plugs with single use scratch pad.

  - Rinse bowl, lid, blades, discharge disc, and discharge chute from the top to the bottom with hot, low pressure water to remove the excess food residues.

  - Foam bowl, lid, blades, discharge disc and discharge chute with Enrich 299 at the recommended concentration.

  - Handwipe everything in 5.7 with a single use scratch pad or white bristled brush. For Safety , put cutting gloves and use handheld white brush to brush the blades.

For Koch Kitchen carts,

  - Rinse Koch Kitchen carts with hot, low-pressure water to remove the excess food residues.

  - Foam Koch Kitchen carts with Enrich 299 at the recommended concentration.

  - Rinse everything in 5.7 to 5.12 from the top down with hot, low-pressure water to remove chemical residues

(Remove excess water inside Koch Kitchen carts)

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize everything in 5.7 with Multi-Quat at the recommended concentration. (200ppm-400ppm)

(Remove excess water inside Koch Kitchen carts)

  - Return power to the Seydelmann

  - As part of pre-op process insert drain plugs

- Procedure (Weekly)

  - On a weekly and or as needed basis, maintenance shall disassemble the blade assembly to allow a thorough foaming and hand cleaning of the shaft and the mating surfaces between blades.  Blades are then sharpened in the maintenance shop.

  - Maintenance reinstalls blades in correct configuration.

  - Remove two drain plugs in bottom of bowl rinse handwipe drain plugs with single use scratch pad.

  - Rinse bowl, lid, blades, discharge disc, and discharge chute from the top to the bottom with hot, low pressure water to remove the excess food residues.

  - Foam bowl, lid, blades, discharge disc and discharge chute with Enrich 299 at the recommended concentration.

  - Handwipe everything in 5.7 with a single use scratch pad or white bristled brush. For Safety , put cutting gloves and use handheld white brush to brush the blades.

  - Rinse everything in 5.7  from the top down with hot, low-pressure water to remove chemical residues.

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize everything in 5.7 with Multi-Quat at the recommended concentration. (200ppm-400ppm)

  - Return power to the Seydelmann

- Records

  - Pre-op checklist to include Charm unit swab record.

  - Master Sanitation Plan

- Document History

| REVIEWED: | PROPOSED CHANGE: |
| --- | --- |
| Roland Pilar Dated:12/7/2021. | Removed Indosan 485 changed Multi-Quat. Removed Soak drain plug and After 30 minutes. Added Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed) |
| Roland Pilar / Alfredo Robles 03/15/2023 | Added SSOP number. |