- Purpose

  - To ensure that appropriate cleaning and sanitizing of the reverse osmosis and ultrafiltration system.

- SCOPE

  - The reverse osmosis and ultrafiltration and other equipment associated with the whey concentrate process.

- procedure

  - Check to make sure that there is an adequate amount of chemical in the chemical supply barrels.

  - Immediately following production, rinse the system to drain until the discharge is clear.

  - Shut down the system and prepare unit for CIP cleaning. Change in-line filters and position hand valve so the cleaning solution flows through the filters.

  - Pre-rinse to drain until the system is clear.

  - Establish recirculation for the wash.

  - Wash Concentrations:

    - Acid Wash – use 5.0 gallons of Ultrasil 75 – 240 second feed time – Concentration of 6,000 – 8,000 ppm with a pH range of 2.3 to 2.5.

    - Alkaline wash – use 5-6 gallons of Ultrasil 63 – 360 second feed time – measured by volume and adjust pH to between 10.0 and 10.8 by adding 50% caustic (~46 ounces) by hand to the system.

  - Circulate the system for 30 minutes at 120oF in both alkaline and acid wash cycles.

  - Rinse to drain using 100 – 120oF water between wash cycles.

  - Add 4.0 gallons of Ultrasil MP – Concentration 5,560 - 8,340 ppm to achieve a pH of 3.6 to 3.9.

  - DO NOT rinse Ultrasil MP from the system – Shut down the system with the Ultrasil MP solution in the system.

- REQUIRED RECORDS

Records maintained per the DFA Record Retention Policy

| Record(s) | Function Responsible |
| --- | --- |
| CIP Charts | Local QA Team |
| Cleaning and Sanitation Records | Local QA Team |

- DOCUMENT REVIEW

| Action | Position Title | Name(s) |
| --- | --- | --- |
| Change Initiator | Quality Assurance Manager | Kellie Clann |
| Author | Quality Assurance Manager | Kellie Clann |
| Approver | Plant Quality & Compliance Manager | Kellie Clann |

- CHANGE HISTORY

Change history, annual review logs and approval dates reside within the DB SharePoint site.

| Date | Change Initiator | Brief Description of Changes Made |
| --- | --- | --- |
| 05/01/23 | Kellie Clann | New. |