- Purpose: The purpose of the Sanitation SOP’s is to provide Miller sanitation personnel the procedures to safely and effectively clean the facility and equipment.

- Scope: Townsend BL2800 Peeler

- Definitions

  - Casings Catch: Compartment where cellulose casings from the peeler are captured.

- Responsibilities

  - Sanitarian: Clean, rinse, and sanitize the equipment

  - Maintenance: Hand clean controls and vacuum pump cabinet quarterly.

  - Pre-op Personnel: Visually inspect, swab, reclean if necessary

  - VP of Administration: Assure adherence to sanitation SOP’s and related documentation.  Review and update as necessary.

- Procedure

  - Put on the appropriate PPE for the sanitation chemicals in use

  - Unplug peeler and shut off steam.

  - Remove peeled casings from casings catch, put in bag, and dispose.

  - Remove casings vacuum hose from peeler. Soak in Enrich 299 at the recommended concentration.

  - Remove casing, slitter assembly.

  - Soak slitter assembly in Enrich 299 at the recommended concentration.

    - After 30 minutes, hand scrub with single use scratch pad or white bristled brush.

    - Rinse all parts with hot, low pressure water.

    - Soak all parts overnight in Multi-Quat at the recommended concentration

  - Scrape major debris from peeler, casing catch, and discharge conveyor with stainless putty knife or similar.

  - Rinse peeler, casing catch and discharge conveyor with low pressure, hot water from the top to the bottom.

  - Foam peeler, casing catch and discharge conveyor with Enrich 299 at the recommended concentration.

  - Handwipe everything in 5.9 with a single use scratch pad or white bristled brush.

  - Rinse everything in 5.9 with low pressure, hot water.

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize everything in 5.9 with Multi-Quat at the recommended concentration.

  - Visually inspect, swab, and reclean if necessary.

- Weekly Perasan A Mixing Procedure & measurements.

    - To mix please use the following based on 25 gallons of water:

    - Perafoam- Mix 6 ounces every 6 gallons of water. (20 ounces total)

    - Perasan A- Mix 1 ounces for every 6 gallons of water (4 ounces total)

    - Titrate using the Perasan A testing procedure, target is 91-195ppm of PerasanA .(+/- 13ppm)

    - Sanitize everything in 5.9 with Perasan A

    - Visually inspect, swab, and reclean if necessary.

- Records

  - Pre-op checklist to include Charm unit swab record.

  - Master Sanitation Plan

- Document History

| REVIEWED: | PROPOSED CHANGE: |
| --- | --- |
| Roland Pilar Dated:12/6/2021 | Removed Indosan 485 changed to Multi-Quat. Added Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed) |
| Roland Pilar 6/27/2022 | 6.0 Weekly Perasan A Mixing Procedure & measurements. 6.1.1-To mix please use the following based on 25 gallons of water: 6.1.2-Perafoam- Mix 6 ounces every 6 gallons of water. (20 ounces total) 6.1.3-Perasan A- Mix 1 ounces for every 6 gallons of water (4 ounces total) 6.1.4-Titrate using the Perasan A testing procedure, target is 91-195ppm of PerasanA 6.1.5-Sanitize everything in 5.9 with Perasan A 6.1.6-Visually inspect, swab, and reclean if necessary. |
| Roland Pilar 9/28/2022 | Added Perasan A mixing procedure & measurement. |
| Roland Pilar / Alfredo Robles 03/15/2023 | Added SSOP number |