- Purpose: The purpose of the Sanitation SOP’s is to provide Miller sanitation personnel the procedures to safely and effectively clean the facility and equipment.

- Scope: U-MEC Combo Weighing Conveyor

- Definitions

  - None

- Responsibilities

  - Sanitarian: Clean, rinse, and sanitize the equipment

  - Pre-op Personnel: Visually inspect, swab, reclean if necessary

  - VP of Administrations: Assure adherence to sanitation SOP’s and related documentation.  Review and update as necessary.

- Procedure

  - Put on the appropriate PPE for the sanitation chemicals in use

  - Turn off power at disconnect on wall and lockout.

  - Bag Rice Lake scale control panel, disconnect on the wall, and motor under conveyor.

  - Scrape major debris from sides, frame, conveyor, discharge chute, and lift with stainless putty knife or similar.

  - Rinse sides, frame, conveyor, discharge chute, and lift from the top to the bottom with hot, low pressure water.

  - Foam sides, frame, discharge chute, and lift with Enrich 299 at the recommended concentration.

  - Rinse everything in 5.6 from the top down with hot, low pressure water to remove chemical residues.

  - Sanitize everything in 5.9 with Multi-Quat at the recommended concentration.

  - Return power to the to the U-Mec.

  - (Weekly) Remove conveyor and soak in Koch kitchen cart with Enrich 299 at the recommended concentration for 30 minutes

  - After 30 minutes dump the Enrich 299 from Koch kitchen cart and rinse conveyor. Reattach the conveyor to the frame and re-rinse the entire machine.

  - Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed)

  - Sanitize everything with Multi-Quat at the recommended concentration.

- Records

  - Pre-op checklist to include Charm unit swab record.

  - Master Sanitation Plan

- Document History

| REVIEWED: | PROPOSED CHANGE: |
| --- | --- |
| Roland Pilar 12/7/2021. | Removed Iodosan 485. Cleaned U-Mec Conveyor weekly. Added Visual inspection using flashlight to ensure the cleanliness of equipment’s and parts before sanitizing. (If found food residues, reclean as needed) |
| Roland Pilar/ Alfredo Robles | Added SSOP number |